# Fan Removal

  - Using the #2 Phillips, remove the (8) Sheet Metal screws and washers holding the fan plate in place, circled in RED, and set them aside for reinstallation later.              
                                                                                     
Figure 1

DO NOT REMOVE THE (4) SCREWS CLOSEST TO THE FANS!

  - Once the screws are removed, lift the fan plate up on one of the long sides of the radio.  Reach in and GENTLY wiggle the fan connector from side to side while lifting up to remove it.              
Figure 2

  - Set the fan assembly and screws aside for installation later.