# Fan Removal

  - Using the #2 Phillips, remove the (4) Sheet Metal screws and washers holding the fan plate in place and set them aside for reinstallation later.              
                                                                                     
Figure 1

DO NOT REMOVE THE (4) SCREWS CLOSEST TO THE FAN!

  - Once the screws are removed, lift the fan plate up, from either side of the radio, just enough for you to reach in and GENTLY wiggle the fan connector from side to side while lifting up to remove it.              
Figure 2a				         Figure 2b

  - Set the fan assembly and screws aside for installation later.