# Side Panel and Top/Bottom Cover removal

- Remove the side panels.

  - If you look closely at the side rails you’ll notice little indentations in them.  This is to allow you to remove the side panels.       
Figure 1

  - Using the flat blade screwdriver GENTLY pry the side panels off the radio.      
Figure 2a				   Figure 2b

  - Set them aside for later reinstallation.

- Remove the Top and Bottom panels.

  - Using a T-20 Torx driver, remove the (4) screws fastening the top cover to the chassis.

Figure 3

  - Using your hands, place your thumbs on the dark grey side support pieces and using your fingers grip the lip of the panel and flex it out slightly, while lifting up.  You may need to push down with your thumbs for added assistance.  Once it ‘pops’ loose, do the same to the other side.             
Figure 4

  - Carefully turn the radio over.

  - Repeat steps ‘a’ and ‘b’, above, to remove the bottom cover.