# RFIO Removal

Figure 1 – Connector side of the HRFIO board.

- Remove the Side Panels and Top cover.  Refer to section covering the removal of the Side Panels and Top/Bottom cover.

- Looking at the HRFIO board, from inside the radio, remove the BNC coaxial cable connecting the HPA to the HRFIO board.

- Disconnect the flat ribbon cable.

- Disconnect the two small, thin coaxial cables going to J2 “RX1 IN” and J15 “XVTX”.  They are located in the upper left of the board.  If you have a RX2 installed, remove the small, thin coaxial cable going to J14 “RX2 IN”.  Be sure to observe what cable went where.  They will be color coded and the colors will vary.  This will be VERY important for reassembly!
Figure 2 – Coaxial connections

- Referring to Figure 1, remove the (8) screws on both sides of ANT 1, ANT 2, ANT 3, with a #1 Phillips head screwdriver.

- Again, referring to Figure 1, remove the nut and washer on each of the (6) BNC connectors using a 9/16” socket.   BE CAREFULL or you may scratch the panel.  I suggest using just enough force with the socket to loosen the nut and back it out the rest of the way with you fingers.

- Carefully slide the HRFIO board out from the Back panel.